SIMATS ENGINEERING

ASSESSMENT TOOL

COURSE CODE:ECA1803                   COURSE NAME: Optical Fiber Communication

Standards-Compliant Laser Transmitter Design, Wavelength Control and Nonlinearity Management for an 8-Channel DWDM Link

Course Outcome – CO2

Analyze the operation and principles of optical sources including LEDs and laser diodes (FP, DBR, DFB, VCSEL, QD), along with their characteristics such as modulation dynamics, spectral stability, efficiency, and the impact of nonlinear effects (SPM, XPM, FWM, SBS, SRS), soliton transmission, and photonic integration on system performance.

Nature of Assessment Problem Solving

Assignment Duration-1 Week

Maximum Marks-100

Primary Bloom’s Levels K3 – Apply | K4 – Analyze

Code of Conduct:

I S HARIPRIYA (192412448) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Priya

Assumptions and Conventions

The problem statement fixes most of the numerical data, but a handful of quantities that any real design depends on are not given. These are stated once here so that every later calculation can be traced back to them. Where a different but reasonable value is used, the conclusions below shift only in the margins quoted and not in the final decision.

Table 0: Assumed parameters not given in the problem statement

| Symbol | Assumed value | Why it is needed |
| --- | --- | --- |
| D (SMF dispersion) | 17 ps/(nm·km) at 1550 nm (β2 ≈ –21.7 ps²/km) | Dispersion broadening, XPM walk-off, FWM phase mismatch. Consistent with the given ¦β2¦ = 20 ps²/km, which corresponds to D ≈ 15.7 ps/(nm·km). |
| ng, cavity length | 3.5 and 300 µm | Fabry–Pérot mode spacing. |
| Brillouin gain bandwidth ΔνB | 20 MHz | Effect of linewidth on the SBS threshold. |
| EDFA noise figure | 5 dB | OSNR estimate. |
| Modulator + coupling + AWG loss | 6 dB combined | Power budget between laser and fibre. |
| Current-tuning coefficient | 0.01 nm/mA (typical DFB) | TEC versus current tuning comparison. |
| Eye-safety reference limit | 10 mW (+10 dBm) for unrestricted, Class 1-type access at 1550 nm | ANSI Z136 check (the exact limit depends on classification and exposure duration). |
| Forward voltage of laser | 1.5 V | Wall-plug efficiency only. |
| Chirp factor αH | ≈ 4 for a directly modulated DFB; ≈ 0–1 for an EAM | Chirp comparison. |

STAGE 1 – INTERPRET THE COMMUNICATION REQUIREMENT

## 1.1 Working backward from the link

The aggregate payload is 8 × 25 Gb/s = 200 Gb/s carried over 6 × 80 km = 480 km. On a 100 GHz grid the eight carriers occupy 7 × 100 GHz = 700 GHz of optical bandwidth, which at 1550 nm is 7 × 0.8 nm = 5.6 nm. The link figure of merit is therefore not the laser alone but the whole chain that the laser starts.

Three numbers drive everything that follows:

- Bit period. Tb = 1/Rb = 1/(25 × 109) = 40 ps. Anything that smears a pulse by a fraction of 40 ps (dispersion, relaxation-oscillation ringing, chirp) shows up as inter-symbol interference.

- Span loss. 0.2 dB/km × 80 km = 16 dB per span. Each EDFA must restore 16 dB, and each adds amplified spontaneous emission (ASE). Six spans accumulate the ASE of six amplifiers.

- Channel pitch versus signal width. An NRZ spectrum has a main-lobe null-to-null width of 2Rb = 50 GHz ≈ 0.4 nm, which is half of the 100 GHz pitch. Whatever the source adds on top of this (linewidth, chirp, side modes, drift) has to fit into the remaining ≈ 50 GHz without leaking into a neighbour.

1.2 Why high speed, narrow linewidth and high power are all mandatory

High modulation speed. With B ≈ 0.7Rb = 0.7 × 25 = 17.5 GHz, the source or its modulator must have a 3 dB bandwidth above 17.5 GHz with a flat response and little ringing. An LED, whose bandwidth is set by the carrier lifetime, cannot meet this (Stage 2 shows ≈ 20 MHz).

Narrow linewidth. A broad or chirped spectrum is penalised three times over. First, in the fibre, a pulse of spectral width Δλ is broadened by Δτ = D·L·Δλ. For a transform-limited 25 Gb/s NRZ signal, Δλ ≈ Rbλ²/c = (25 × 109)(1550 × 10-9)²/(3 × 108) ≈ 0.2 nm, so one 80 km span gives Δτ = 17 × 80 × 0.2 ≈ 272 ps, which is 6.8 bit periods. Second, a wide spectrum or strong side modes spills into the adjacent channel at only 0.8 nm separation and produces linear crosstalk. Third, a narrow-linewidth carrier lowers the SBS threshold (Stage 4), so linewidth must be controlled in both directions: narrow at the laser, deliberately broadened at the fibre input.

High output power. The optical signal-to-noise ratio of a chain of N identical spans is OSNR (dB, 0.1 nm) ≈ 58 + Pch – Lspan – NF – 10 log N. With Pch = +3 dBm (2 mW), Lspan = 16 dB, NF = 5 dB and N = 6:

OSNR ≈ 58 + 3 – 16 – 5 – 10 log(6) = 58 + 3 – 16 – 5 – 7.78 ≈ 32.2 dB

Each 1 dB of lost launch power costs 1 dB of OSNR, so the source must supply enough power, after modulator, coupling and multiplexer losses, to reach about +3 dBm per channel. This is comfortably above the roughly 20–24 dB typically needed for 25 Gb/s on–off keying (assumed), which leaves room for nonlinear penalty, connectors and ageing. But the power cannot simply be raised, because of SBS, SPM and eye safety. The design is therefore a balance, not a maximisation.

1.3 Source characteristics that set wavelength, efficiency and spectral purity

- Bandgap E_{g}: sets the emission wavelength, λ(µm) ≈ 1.24/Eg(eV). The active region must be an InGaAsP/InGaAlAs alloy with Eg ≈ 0.80 eV.

- Internal quantum efficiency η_{int}: the fraction of injected carriers that recombine radiatively, ηint = τnr/(τr + τnr). It controls how much current becomes light.

- Threshold current I_{th} and slope efficiency η_{s}: P = ηs(I – Ith) fixes the power available at a given bias, and Ith governs bias current, heat and chirp.

- Linewidth and side-mode suppression ratio (SMSR): set the spectral purity needed on a 0.8 nm grid and the SBS threshold.

1.4 Modulation, chirp and temperature quantities to be controlled

For reliable 25 Gb/s operation the design must control the relaxation-oscillation frequency fR and damping (they set the usable bandwidth and eye overshoot), the linewidth-enhancement (chirp) factor αH and the resulting transient and adiabatic chirp, the extinction ratio, and the temperature coefficient of wavelength (0.1 nm/°C). The last matters most on a dense grid: a 40 °C ambient swing moves an uncontrolled laser by 4.0 nm, which is five full channel spacings.

1.5 Nonlinear quantities that become important

With eight channels, six cascaded spans and 2 mW per channel (16 mW total), the quantities that matter are the effective length Leff (which controls how far the nonlinearity acts), the nonlinear phase φNL = γPLeff accumulated over spans (SPM, and XPM from the seven other channels), the channel-spacing and dispersion dependent phase mismatch (FWM), and the SBS and SRS threshold powers compared with the per-channel and total launch power.

1.6 Role of the standards

Table 1: Role of the applicable standards in defining the transmitter specification

| Standard | What it contributes to the specification |
| --- | --- |
| ITU-T G.957 | Optical interface parameters for SDH-class equipment at 1550 nm: mean launched power range, central wavelength and spectral width, SMSR (≥ 30 dB), extinction ratio and eye-mask/pulse-shape limits, and the receiver sensitivity/overload that the launch power must support. |
| IEEE 802.3bs / 802.3bm | Ethernet PHY transmitter requirements at 25 Gb/s per lane (100G/200G/400G): per-lane launch power and OMA, extinction ratio, SMSR, RIN, wavelength window, transmitter quality measured on an eye mask or TDECQ, and the 25.78125 GBd lane rate. |
| ANSI Z136 (Z136.1/Z136.2) | Laser safety: maximum permissible exposure and hazard classification, which limit the total optical power that may be accessible at an open connector or broken fibre and require control measures above the Class 1 level. |
| ITU-T G.694.1 (additional) | The 100 GHz DWDM frequency grid anchored at 193.1 THz, which defines the eight target wavelengths. |

1.7 System parameters influencing soliton or DSP compensation and integration

Soliton transmission depends on the dispersion length LD = T0²/|β2|, the nonlinear length LNL = 1/(γP0), the soliton order N and the amplifier spacing relative to LD. DSP compensation depends on the receiver being coherent, the number of channels that must be processed jointly, and the ADC bandwidth. The integration decision depends on footprint, power, coupling loss, wavelength accuracy across eight lanes, reliability and volume.

1.8 Characteristics an acceptable transmitter must possess

Table 2: Required transmitter characteristics derived from Stage 1

| Characteristic | Requirement (derived) |
| --- | --- |
| Wavelength | Eight carriers on the 100 GHz grid near 1550 nm, each within ±0.1 nm |
| Modulation | ≥ 17.5 GHz electrical bandwidth, low chirp, adequate extinction ratio |
| Spectral purity | Single longitudinal mode, SMSR ≥ 30 dB (G.957), narrow linewidth |
| Power | About +3 dBm (2 mW) per channel at launch, within SBS and eye-safety limits |
| Stability | Wavelength locked over a 40 °C ambient swing |
| Compliance | G.957 / IEEE 802.3bs/bm parameters verifiable; ANSI Z136 safety measures |
| Implementation | Scalable to eight lanes with low footprint and power |

## Problem Requirement Statement

The proposed 8-channel DWDM transmitter must deliver eight single-mode 25 Gb/s NRZ carriers on the 100 GHz grid near 1550 nm, each with a modulation bandwidth of at least 17.5 GHz, SMSR of at least 30 dB, low chirp, and about +3 dBm (2 mW) launch power per channel. Its wavelength must stay within ±0.1 nm over a 40 °C ambient variation. Its launch power, linewidth and dispersion map must keep SPM, XPM, FWM, SBS and SRS penalties small over six 80 km spans (480 km), while meeting ITU-T G.957, IEEE 802.3bs/bm and ANSI Z136 requirements in a compact, scalable implementation.

STAGE 2 – INVESTIGATE AND SELECT THE OPTICAL SOURCE / TRANSMITTER CONFIGURATION

## 2.1 Recombination processes and the role of the bandgap

Injected electrons and holes in the active region recombine in three ways. Radiative (spontaneous) recombination emits a photon of energy ≈ Eg, which is the useful process in an LED and below threshold in a laser. Non-radiative recombination (Shockley–Read–Hall defects, and Auger recombination, which is particularly strong in the narrow-gap InGaAsP alloys used at 1.55 µm) turns the energy into heat. Stimulated recombination dominates above threshold and produces the coherent output. The bandgap fixes the photon energy, and therefore the wavelength.

2.2 Emission wavelength

λ ≈ 1.24 / Eg = 1.24 / 0.80 = 1.55 µm = 1550 nm

This sits exactly in the 1550 nm low-loss window (0.2 dB/km), and in the gain band of erbium-doped fibre amplifiers, so the material choice is consistent with the link. On the ITU grid it corresponds to ≈ 193.4 THz.

2.3 Internal quantum efficiency

The total carrier recombination rate is the sum of the radiative and non-radiative rates:

1/τ = 1/τr + 1/τnr = 1/10 ns + 1/40 ns = 0.100 + 0.025 = 0.125 ns-1  →  τ = 8 ns

ηint = τ/τr = τnr/(τr + τnr) = 40/(10 + 40) = 8/10 = 0.80  (80 %)

The same carrier lifetime explains why an LED is slow: its electrical 3 dB bandwidth is limited to f3dB ≈ 1/(2πτ) = 1/(2π × 8 ns) ≈ 19.9 MHz, which is about 880 times too low for the 17.5 GHz requirement.

A useful cross-check uses the given slope efficiency. Since P/I = ηd·(hν/q) and hν/q = 0.80 V, the differential quantum efficiency is ηd = 0.3 mW/mA ÷ 0.80 V = 0.375. The ratio ηd/ηint = 0.375/0.80 = 0.47 is the fraction of stimulated photons that escape through the facets (αm/(αi + αm)), which is a plausible value for a cleaved or coated cavity. The given data are therefore self-consistent.

2.4 Comparison of candidate sources

Typical representative values for 1550 nm devices are used (assumed from standard device literature, not given in the problem). The LED spectral width follows Δλ ≈ 1.8kTλ²/(hc) = 1.8(0.0259 eV)(1.55 µm)²/(1.24 µm·eV) ≈ 90 nm.

Table 3: Comparison of LED, FP, DBR, DFB, VCSEL and QD sources for 25 Gb/s DWDM

| Source | Fibre-coupled power | Spectral width / SMSR | Modulation speed | Efficiency / threshold | Fibre coupling | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| LED | ≤ 0.1 mW (–10 dBm or less) | ≈ 90 nm, no SMSR | ≈ 20 MHz (τ = 8 ns) | Low (spontaneous only); no threshold | Poor (wide, Lambertian beam) | Rejected: ≈ 113 times wider than a channel |
| FP laser | 1–10 mW | 1–5 nm, many modes; mode spacing ≈ 1.14 nm | ≈ 10–20 GHz | Good; ≈ 10–30 mA | Moderate | Rejected: modes fall in neighbouring 0.8 nm channels; partition noise |
| DBR laser | 1–10 mW | Single mode, 1–10 MHz, SMSR 35–45 dB | ≈ 5–10 GHz | Good, passive-section loss; ≈ 15–30 mA | Good | Credible; mode hops, tuning-section complexity |
| DFB laser | 5–20 mW | Single mode, 0.1–10 MHz, SMSR 35–50 dB | 10–30 GHz (20 GHz given) | Good (ηd = 0.375 here); 12 mA here | Good, isolator-friendly | Selected |
| VCSEL (1550 nm) | 0.5–2 mW | Single mode, narrow | ≈ 5–10 GHz at 1550 nm | Moderate, thermal roll-over; 1–5 mA | Excellent, circular beam | Rejected: too little power for six spans |
| QD laser | 5–15 mW | Narrow gain, low linewidth, small αH | 10–25 GHz | Good, low temperature sensitivity; ≈ 5–15 mA | Good | Credible; lower maturity at 1550 nm |

2.5 Threshold condition and side-mode suppression

Laser oscillation begins when the round-trip modal gain equals the round-trip loss (amplitude condition) and the round-trip phase is a multiple of 2π (phase condition):

Γ gth = αi + (1/2L) ln(1/R1R2)        and        2βL = 2πm

In a Fabry–Pérot cavity the longitudinal mode spacing is Δλ = λ²/(2ngL) = (1.55 × 10-6)²/(2 × 3.5 × 300 × 10-6) ≈ 1.14 nm. Because this is larger than the 0.8 nm channel pitch while the gain spectrum is several nm wide, several FP modes would oscillate, some of them inside neighbouring channels. A DFB laser puts a Bragg grating (λB = 2neffΛ/m) inside the cavity, which gives different threshold gains to different modes, so a single mode lases and the others are suppressed by the SMSR (≥ 35 dB typical). On a DWDM link, a side mode that is 35 dB down but lands on a neighbouring channel contributes about –35 dB of linear crosstalk. Summed over several interferers it can approach –26 dB, which is why SMSR ≥ 30 dB (G.957) is treated as a hard requirement. Linewidth enters similarly: the narrower the line, the less of the 50 % guard fraction is used up.

2.6 Source selection

The DFB laser is the only candidate that satisfies all four primary requirements at once:

Table 4: Justification of the DFB laser against at least four technical parameters

| Parameter | Requirement | DFB laser (calculated / typical) | Met? |
| --- | --- | --- | --- |
| Emission wavelength | 1550 nm window | λ = 1.24/0.80 = 1550 nm | Yes |
| Internal quantum efficiency | High (low heat, efficient bias) | ηint = 0.80 | Yes |
| Output power | ≈ 2 mW per channel after ≈ 6 dB losses | 8.4 mW at 40 mA (9.24 dBm), Stage 3 | Yes |
| Spectral purity | Single mode, SMSR ≥ 30 dB | SMSR 35–50 dB, linewidth ≈ MHz | Yes |
| Modulation bandwidth | ≥ 17.5 GHz | 20 GHz (direct); modulator-limited if external | Yes (2.5 GHz margin) |
| Threshold | Low, to limit bias and heat | 12 mA | Yes |

Credible alternative. A tunable DBR laser (for example a sampled-grating DBR) would become preferable if the network required wavelength-agile, reconfigurable channel assignment, or a single sparable part replacing eight fixed-wavelength DFB inventory codes. In that case its tunability outweighs the mode-hop and control complexity. A QD laser would be the next alternative if uncooled operation (no TEC) became the dominant cost and power concern, because its characteristic temperature is high and its chirp factor is small.

DESIGN DECISION 1 – SELECT THE PROPOSED OPTICAL SOURCE I recommend a 1550 nm InGaAsP/InP distributed-feedback (DFB) laser diode optical source because it emits at λ = 1.24/0.80 = 1550 nm with ηint = 0.80, delivers 8.4 mW (9.24 dBm) at 40 mA from a 12 mA threshold, gives single-longitudinal-mode operation with SMSR ≥ 30–35 dB and a MHz-class linewidth that fits inside a 0.8 nm channel, and offers 20 GHz bandwidth (2.5 GHz above the 17.5 GHz requirement) with good fibre coupling. LED, FP and 1550 nm VCSEL sources fail on spectral width/speed, side modes and power respectively.

STAGE 3 – DEVELOP AND EVALUATE THE MODULATION, CHIRP AND SPECTRAL-STABILITY STRATEGY

3.1 Output power at the bias current

P = ηs(I – Ith) = 0.3 mW/mA × (40 mA – 12 mA) = 0.3 × 28 = 8.4 mW

P(dBm) = 10 log10(8.4 mW / 1 mW) = 10 × 0.9243 = +9.24 dBm

The electrical input at an assumed 1.5 V is 40 mA × 1.5 V = 60 mW, so the wall-plug efficiency is 8.4/60 = 14 %, with the remaining 51.6 mW dissipated as heat that the TEC must remove. Because the bias is 28 mA above threshold, the laser operates in the linear part of its L–I curve.

Power budget (assumed 6 dB of modulator, coupling and AWG loss). If an external modulator is used (Section 3.4), the CW laser power of +9.24 dBm becomes +9.24 – 6 = +3.24 dBm = 2.1 mW per channel at the multiplexer output. This is 0.23 dB above the 2 mW (+3.01 dBm) target, which is a thin margin. A small SOA or booster stage, or a modest bias increase (for example 45 mA gives 0.3 × 33 = 9.9 mW), provides headroom if the actual losses are higher than assumed.

3.2 Bandwidth requirement and margin

B ≈ 0.7 Rb = 0.7 × 25 Gb/s = 17.5 GHz

Margin = 20 GHz – 17.5 GHz = 2.5 GHz  (20/17.5 = 1.14, i.e. 14 % above requirement)

The 20 GHz laser bandwidth is sufficient but not generous. A sensitivity check is worth making: for the Ethernet lane rate of 25.78125 Gb/s, B = 0.7 × 25.78 = 18.05 GHz, and the margin falls to 1.95 GHz (11 %). The design is therefore workable but must not be pushed to higher symbol rates without a faster device. Any package parasitics (bond-wire inductance, driver roll-off) would erode this margin directly, which argues for a short, controlled-impedance RF path.

3.3 Rate equations and relaxation oscillation

The interaction of the carrier density N and photon density S in the active region is described by the single-mode rate equations:

dN/dt = I/(qV) – N/τn – G0(N – Ntr)S / (1 + εS)

dS/dt = Γ G0(N – Ntr)S / (1 + εS) – S/τp + ΓβspN/τn

Here V is the active volume, τn the carrier lifetime (8 ns above), τp the photon lifetime (a few ps), G0 the gain coefficient, Ntr the transparency density, ε the gain-compression factor, Γ the confinement factor and βsp the spontaneous-emission coupling. Linearising about the bias point gives a second-order response with a resonance at the relaxation-oscillation frequency:

fR ≈ (1/2π) √( G0 S0 / τp ),   with S0 ∝ (I – Ith)  →  fR ∝ √(I – Ith)

The 3 dB bandwidth of this response is f3dB ≈ √(1 + √2) fR ≈ 1.55 fR (for negligible damping), so the given 20 GHz implies fR ≈ 12.9 GHz at 28 mA of excess current. Raising the bias raises fR only as the square root, so doubling the speed would take about four times the excess current, which is thermally impractical.

Small-signal versus large-signal. In small-signal modulation the response is linear and has a resonance peak at fR that can be damped by gain compression. In large-signal NRZ modulation the laser swings from near threshold to well above it, so the instantaneous fR changes with the photon density. The result is turn-on delay, ringing (overshoot) in the time domain, pattern-dependent jitter, and a transient chirp that peaks at the edges. The small-signal bandwidth is therefore an optimistic bound, and a 20 GHz value gives only a modest large-signal eye margin at 25 Gb/s.

3.4 Direct versus external modulation

Chirp (frequency modulation accompanying intensity modulation) of a laser is described by:

Δν(t) = (αH/4π) [ d ln P(t)/dt + κ P(t) ]

The first term is transient chirp, strongest at the edges, and the second is adiabatic chirp, which makes the ‘1’ level sit at a different frequency from the ‘0’ level. With αH ≈ 4, a directly modulated DFB broadens the optical spectrum well beyond the ≈ 0.2 nm transform-limited value. Because dispersion converts frequency content into delay, this chirp interacts with the fibre. Depending on the sign of D, it can briefly compress pulses and then broaden them quickly.

To see the scale of the dispersion problem independent of chirp, with transform-limited Δλ ≈ 0.2 nm, the per-span accumulation is 17 × 80 = 1360 ps/nm, giving Δτ ≈ 272 ps. Allowing only Tb/4 = 10 ps of broadening, the uncompensated reach is L ≈ 10/(17 × 0.2) ≈ 2.9 km. This shows that dispersion compensation is mandatory in any 25 Gb/s NRZ design over 80 km spans, and that a chirped source makes the problem worse.

Table 5: Direct modulation versus external modulation

| Aspect | Direct modulation (DML) | External modulation (EAM / MZM) |
| --- | --- | --- |
| Chirp | αH ≈ 4: strong transient + adiabatic chirp | EAM α ≈ 0–1; push-pull MZM ≈ 0 (nearly transform-limited) |
| Bandwidth | Limited by fR: 20 GHz gives only 2.5 GHz margin | Set by modulator (≥ 20–30 GHz for EAM); laser runs CW |
| Dispersion interaction | Chirp-dependent, penalty rises quickly beyond a few km | Spectrum ≈ 0.2 nm; predictable, compensable with DCF |
| Transmission distance | Short reach (metro/data-centre) | Long reach, multi-span |
| Wavelength stability | Bias changes with data: dynamic wavelength shift | Constant bias: stable, trackable wavelength |
| Cost / complexity | Lowest | Higher, extra insertion loss (≈ 3–6 dB assumed) |
| SBS | Modulation broadens spectrum | Needs deliberate carrier dithering (Stage 4) |

## 3.5 Wavelength drift and TEC stability

Δλdrift = (0.1 nm/°C) × (40 °C) = 4.0 nm

Δλdrift / Δλchannel = 4.0 nm / 0.8 nm = 5 channel spacings

An uncontrolled laser would therefore wander across five neighbouring channels, which is clearly unacceptable. For the ±0.1 nm tolerance, the required temperature stability follows from:

ΔTmax = Δλtol / (dλ/dT) = 0.1 nm / (0.1 nm/°C) = ±1.0 °C

This ±1.0 °C is the absolute limit, with the entire tolerance consumed by temperature alone. Since the tolerance must also cover aging, locker accuracy and current-induced shifts, the TEC loop should be specified at ±0.1 °C, which corresponds to ±0.01 nm (10 times better than the limit) and is routinely achieved with a thermistor and PID controller.

Table 6: TEC control versus injection-current control for wavelength tuning

| Aspect | TEC (temperature) control | Injection-current control |
| --- | --- | --- |
| Tuning coefficient | 0.1 nm/°C (given) | ≈ 0.01 nm/mA (assumed typical) |
| Range | 4 nm over 40 °C: covers the full drift | 0.1 nm needs ≈ 10 mA: very limited range |
| Side effect on power | None on optical power | 1 mA changes P by 0.3 mW (3.6 %, 0.15 dB); 10 mA changes it by 3 mW |
| Side effect on dynamics | None | Changes fR, chirp and extinction ratio |
| Speed | Slow (seconds): ideal for drift | Fast (µs): suited to fine trim or dither |
| Role in design | Primary wavelength stabilisation | Fine trim only, plus small dither (Stage 4) |

DESIGN DECISION 2 – SELECT THE MODULATION CONFIGURATION I recommend external (electro-absorption, integrated EML-type) modulation and TEC-based wavelength control with an etalon wavelength-locker feedback loop (current used only for fine trim) because the DFB laser delivers 8.4 mW (+9.24 dBm), which still leaves ≈ +3.2 dBm (2.1 mW) per channel after the assumed 6 dB of losses. Direct modulation would leave only 2.5 GHz bandwidth margin and impose αH ≈ 4 chirp on an NRZ signal that already suffers ≈ 272 ps of dispersive broadening per span. Moreover, the uncontrolled 4.0 nm drift (five channels) is reduced to ±0.01 nm by a ±0.1 °C TEC, ten times inside the ±0.1 nm limit.

# STAGE 4 – INVESTIGATE AND CONTROL THE OPTICAL NONLINEARITIES

## 4.1 Effective length and SPM nonlinear phase

The loss coefficient in nepers is α = 0.2 dB/km ÷ 4.343 = 0.04605 km-1, and αL = 0.04605 × 80 = 3.684.

Leff = (1 – e-αL)/α = (1 – e-3.684)/0.04605 = (1 – 0.02512)/0.04605 = 21.17 km

Only the first ≈ 21 km of each 80 km span, where the power is still high, contributes to nonlinear effects. With P = 2 mW = 0.002 W and γ = 1.3 W-1km-1:

φNL (per span) = γ P Leff = 1.3 × 0.002 × 21.17 = 0.0550 rad

φNL (6 spans) = 6 × 0.0550 = 0.330 rad  (≈ 18.9°)

This assumes identical launch power at every span and a coherent, linear accumulation, which is the worst case. The value is well below the commonly used rule-of-thumb limit of about 1 rad, so SPM from a single channel is not the dominant concern. As a check, the nonlinear length is LNL = 1/(γP) = 384.6 km, and the total effective length 6 × 21.17 = 127.0 km gives 127.0/384.6 = 0.330 rad, in agreement.

## 4.2 XPM and FWM

XPM. The phase shift on one channel due to the other seven is up to φXPM = 2γ(ΣPj)Leff, where the factor 2 applies to co-polarised channels. For seven interferers at 2 mW:

φXPM,max = 2 × 1.3 × (7 × 0.002) × 21.17 = 0.771 rad per span (no walk-off)

This worst case is more than ten times the SPM of one channel, which shows that XPM is the larger phase term in a WDM system. In practice, dispersion makes channels walk through each other. The adjacent-channel walk-off per span is D·Δλ·L = 17 × 0.8 × 80 = 1088 ps ≈ 27 bit periods, so the XPM interaction averages out strongly, and its magnitude falls with increasing dispersion and increasing channel spacing. The phase distortion becomes intensity noise through dispersion, and it grows with launch power, with the number of channels, and with decreasing channel spacing.

FWM. Three channels at ωi, ωj, ωk generate a new field at ωi + ωj – ωk. On a uniform grid many of these products land exactly on other channels, and the result is in-band crosstalk. The efficiency depends on the phase mismatch Δβ = (2πλ²/c)·D·Δf1Δf2, which for adjacent 100 GHz channels is:

Δβ = 2π(1550 × 10-9)²(17 × 10-6)(1011)² / (3 × 108) = 8.55 × 10-3 m-1 = 8.55 km-1

The corresponding coherence length is 2π/Δβ ≈ 0.73 km, far shorter than Leff, so the process is strongly phase-mismatched. The FWM efficiency is approximately η ≈ α²/(α² + Δβ²) = (0.04605)²/(0.04605² + 8.55²) ≈ 2.9 × 10-5. For one non-degenerate product (degeneracy factor d = 6) at P = 2 mW per channel:

PFWM = η (dγLeff/3)² P³ = 2.9 × 10-5 × (2 × 27.5)² × (2 × 10-3)³ ≈ 7.0 × 10-10 W ≈ –61.5 dBm

This is ≈ –64.5 dB relative to a +3 dBm channel, per span and per product. If the products added fully coherently over six spans the growth could reach 6² = 36 (15.6 dB), still about –49 dB, and several products land on a middle channel, so the crosstalk stays small for the stated fibre. The result is only as good as the dispersion. In dispersion-shifted fibre (D ≈ 0) Δβ → 0 and FWM becomes severe, and in dispersion-compensating fibre, a lower Aeff and higher γ also raise the penalty. Unequal channel spacing or avoiding the zero-dispersion wavelength are the usual remedies, and standard SMF with D ≈ 17 ps/(nm·km) already provides the key protection.

## 4.3 SBS and SRS thresholds

With Aeff = 80 µm² = 80 × 10-12 m², Leff = 21.17 km = 21 170 m:

Pth(SBS) ≈ 21 Aeff/(gB Leff) = (21 × 80 × 10-12) / (5 × 10-11 × 21 170) = 1.68 × 10-9 / 1.0585 × 10-6 = 1.59 mW (+2.0 dBm)

Pth(SRS) ≈ 16 Aeff/(gR Leff) = (16 × 80 × 10-12) / (6 × 10-14 × 21 170) = 1.28 × 10-9 / 1.270 × 10-9 = 1.008 W (+30 dBm)

Table 7: Threshold powers compared with the launch powers

| Effect | Threshold | Launch power compared | Ratio | Conclusion |
| --- | --- | --- | --- | --- |
| SBS (per channel) | 1.59 mW (+2.0 dBm) | 2 mW per channel (+3.0 dBm) | 2.0/1.59 = 1.26 (+1.0 dB over) | Exceeded by 1.0 dB for a narrow-linewidth carrier; mitigation needed |
| SRS (total) | 1.008 W (+30 dBm) | 16 mW total (+12.0 dBm) | 16 mW / 1008 mW = 1.6 % | Safe: 18 dB below threshold |

This is the key finding of the nonlinear analysis. The 2 mW per-channel launch, which is attractive for OSNR, exceeds the SBS threshold of a narrow-linewidth carrier by 1.0 dB. SBS acts on each channel independently, because the Brillouin gain bandwidth is tens of MHz, so the comparison must be made per channel (2 mW) and not against the total. SRS, in contrast, depends on the total power across the band, and 16 mW is only 1.6 % of 1.008 W. For the 5.6 nm span of the eight channels, the Raman power transfer (a slight tilt favouring longer wavelengths) is negligible.

## 4.4 SBS versus SRS and the effect of linewidth

Table 8: Comparison of SBS and SRS

| Aspect | SBS | SRS |
| --- | --- | --- |
| Physical origin | Interaction with acoustic phonons (electrostriction) | Interaction with optical phonons (molecular vibrations) |
| Scattered direction | Backward only (in single-mode fibre) | Forward and backward |
| Frequency shift | ≈ 11 GHz (≈ 0.09 nm) | ≈ 13 THz (≈ 100 nm) |
| Gain bandwidth | ≈ 20–50 MHz: extremely narrow | ≈ 5–7 THz: very broad (≈ 40 nm) |
| Gain coefficient | gB = 5 × 10-11 m/W | gR = 6 × 10-14 m/W (about 800 times lower) |
| Threshold (this link) | 1.59 mW per channel | 1.008 W |
| Power transfer | Back-reflects signal power to the transmitter | Transfers power from shorter to longer wavelength channels |
| Effect on DWDM | Limits power per channel, causes noise and reflection | Causes gain tilt and bit-pattern dependent crosstalk between channels |

Effect of source linewidth. The SBS threshold increases with the linewidth of the signal: Pth ≈ Pth0(1 + Δνs/ΔνB). The 1.59 mW value is for a source much narrower than the Brillouin bandwidth, so a good narrow-linewidth DFB is the worst case for SBS even though it is the best case for crosstalk and dispersion. With the assumed ΔνB = 20 MHz, a source broadened to Δνs = 200 MHz would give:

Pth(SBS) ≈ 1.59 mW × (1 + 200/20) = 1.59 × 11 = 17.5 mW (+12.4 dBm)  →  margin over 2 mW = 9.4 dB

## 4.5 Eye safety and mitigation of nonlinear penalty

Total launch power is 8 × 2 mW = 16 mW = 10 log(16) = +12.04 dBm. Taking the assumed 10 mW (+10 dBm) limit for unrestricted, Class 1-type access at 1550 nm, the aggregate exceeds it by 2.04 dB, while each individual channel (2 mW) does not. The transmitter output therefore cannot be treated as inherently eye-safe at an open connector. This is not unusual for amplified DWDM systems: the standard response (ANSI Z136.2 together with ITU-T G.664) is to place the equipment in a controlled-access hazard class, with automatic laser shutdown (ALS) or automatic power reduction on fibre break, keyed or shuttered connectors, warning labels and trained-personnel procedures. If an unrestricted Class 1 classification were mandatory, the per-channel power would have to be limited to 10 mW/8 = 1.25 mW (+0.97 dBm). That reduction would also bring SBS below threshold (1.25 < 1.59 mW), at a cost of 10 log(2/1.25) = 2.04 dB of OSNR (32.2 dB falls to 30.2 dB).

Two practical methods reduce the nonlinear penalty without cutting the received signal level:

- Carrier phase dithering / linewidth broadening. A small tone applied to the laser current or a phase modulator at ≥ 100 MHz (above the acoustic damping rate) broadens the effective linewidth to ≈ 200 MHz and raises the SBS threshold to ≈ 17.5 mW. With the assumed 0.01 nm/mA, a dither of only 0.2 mA gives a 0.002 nm (≈ 250 MHz) excursion at a power ripple of about 0.06 mW (0.03 dB), so the signal level is untouched.

- Dispersion management and fibre/amplifier choice. A dispersion map using local dispersion of 17 ps/(nm·km) with periodic compensation (DCF or fibre Bragg grating) per span keeps XPM and FWM phase-mismatched and walk-off high, while hybrid EDFA/Raman amplification or a large-Aeff fibre allows a lower launch power to be held at the same OSNR.

DESIGN DECISION 3 – CREATE THE NONLINEARITY-CONTROL STRATEGY I recommend a combined SBS-dithering, dispersion-managed (per-span DCF), moderate-power nonlinear-control strategy with automatic-laser-shutdown eye-safety protection because the calculated SPM phase is small (φNL = 0.055 rad per span and 0.330 rad over six spans), SRS is 18 dB below threshold (16 mW against 1.008 W), and XPM and FWM are suppressed by the 1088 ps/span walk-off and the 8.55 km-1 phase mismatch at D = 17 ps/(nm·km). SBS is the one effect that is actually violated (1.59 mW threshold against 2 mW), and it is handled by ≥ 100 MHz carrier dithering, which raises the threshold to ≈ 17.5 mW without reducing the received signal. The +12 dBm total exceeds the assumed 10 mW Class 1 limit, so ALS, access control and labelling are part of the design.

# STAGE 5 – EVALUATE SOLITON TRANSMISSION, DIGITAL BACKPROPAGATION AND PHOTONIC INTEGRATION

## 5.1 Physical balance enabling soliton transmission

In the anomalous dispersion regime (β2 < 0, D > 0 at 1550 nm in standard fibre) the leading edge of a pulse travels slower, and the trailing edge faster, which tends to compress a positively chirped pulse. The Kerr nonlinearity produces the opposite effect: self-phase modulation chirps the pulse so that the leading edge is red-shifted. When the SPM-induced chirp exactly counteracts the dispersive chirp, a sech-shaped pulse propagates without changing shape. This is a fundamental soliton, and it requires LNL = LD, i.e. N = 1. The pulse must also be maintained at the correct energy, so every loss must be offset by gain, and the pulse must remain well separated from its neighbours.

## 5.2 Dispersion length, soliton power and soliton order

LD = T0²/|β2| = (15 ps)² / (20 ps²/km) = 225/20 = 11.25 km

For a fundamental soliton LNL = LD, so:

P0 = 1/(γ LD) = 1/(1.3 W-1km-1 × 11.25 km) = 1/14.625 = 0.0684 W = 68.4 mW (+18.3 dBm)

At the actual 2 mW per-channel launch power:

LNL = 1/(γP0) = 1/(1.3 × 0.002) = 384.6 km

N² = LD/LNL = 11.25/384.6 = 0.0293   →   N = 0.171

Table 9: Soliton feasibility assessment

| Quantity | Value | Soliton requirement | Assessment |
| --- | --- | --- | --- |
| Dispersion length LD | 11.25 km | — | — |
| Fundamental-soliton peak power | 68.4 mW (+18.3 dBm) | P0 = 1/(γLD) | 34 times (15.3 dB) above the 2 mW launch |
| Nonlinear length at 2 mW | 384.6 km | LNL = LD = 11.25 km | 34 times too long |
| Soliton order N | 0.171 | N = 1 | ≪ 1: dispersion dominates, no soliton |
| Soliton period z0 = (π/2)LD | 17.7 km | Amplifier spacing ≲ LD | 80 km span = 7.1 LD: too long |
| Pulse spacing in bit slot | 40 ps / 15 ps = 2.7 T0 | ≳ 5–6 T0 to avoid interaction | Too close: solitons would attract/repel |
| Average power of 8 channels (RZ, mean over random data) | ≈ 25 mW per channel (E = 2P0T0 ≈ 2.05 pJ), ≈ 205 mW total | Within eye-safety and SBS limits | Far above the 10 mW and 16 mW limits |

Decision on soliton transmission: not feasible. At 2 mW, N = 0.17, so the pulse spreads as it does in a linear dispersive medium, because nonlinearity is a perturbation over a 384.6 km nonlinear length. Making it a soliton would need 68.4 mW peak power per channel, a return-to-zero format with pulses of 15 ps width at 40 ps spacing (interaction-prone), amplifiers spaced every 11 km rather than 80 km, and an average power of about 205 mW across eight channels. This breaks the SBS, eye-safety and amplifier-spacing constraints. Moreover, WDM collisions and Gordon–Haus timing jitter add further difficulty. The link is therefore designed as a quasi-linear, dispersion-managed NRZ system.

## 5.3 Digital backpropagation

Digital backpropagation (DBP) numerically inverts the fibre propagation. The received complex field is propagated backwards through a virtual fibre with the sign of dispersion and nonlinearity reversed, usually by the split-step Fourier method, with a linear step (a dispersion filter, e-jβ_{2ω²Δz/2}) alternating with a nonlinear step (a phase rotation, e-jγ|E|²L_{eff}). In this way both chromatic dispersion and deterministic SPM (and, if the channels are processed jointly, XPM) can be removed.

Practical limitation. DBP needs access to the full complex optical field, which requires a coherent receiver, and compensates only deterministic effects: signal–ASE nonlinear interaction, PMD and transceiver noise remain. For multi-channel operation the compensation is effective only for the channels that are jointly processed. Handling all eight channels would require ≈ 0.8 THz of joint bandwidth and ADC/DSP speeds far beyond practical limits, with complexity growing with the number of steps per span times six spans. Single-channel DBP leaves the XPM of the other seven channels uncompensated, so the net gain is typically of the order of 1 dB only.

Appropriateness for this link. The system here uses intensity-modulated, direct-detected 25 Gb/s NRZ, where the phase is discarded at the photodiode, so full DBP is not applicable. It is therefore not appropriate for the complete 8-channel link. A realistic role for electronics is limited to receiver-side equalisation (FFE/MLSE) and FEC, with dispersion handled optically (DCF per span).

## 5.4 Standards compliance

Table 10: Transmitter parameters specified by ITU-T G.957 and IEEE 802.3bs/bm and how they are checked

| Parameter | Standard | Proposed check / design value |
| --- | --- | --- |
| Mean launched power | G.957, 802.3bm/bs (per lane) | Optical power meter after AWG: 2.1 mW per channel (+3.2 dBm) with APC |
| Central wavelength and window | G.957, G.694.1, 802.3 | Optical spectrum analyser / wavemeter: each λ within ±0.1 nm of 193.1–193.8 THz grid |
| Spectral width and SMSR | G.957 (SMSR ≥ 30 dB) | OSA at 0.01 nm resolution: target SMSR ≥ 35 dB |
| Extinction ratio and eye mask | G.957, 802.3 (ER, eye mask, TDECQ) | Sampling oscilloscope with reference receiver, PRBS31, mask margin |
| OMA and RIN | 802.3bm/bs | OMA from eye measurement; RIN with low-noise receiver |
| Rise/fall time and jitter | G.957, 802.3 | ≈ 0.35/B = 0.35/17.5 GHz ≈ 20 ps rise time, checked on scope |
| Bit error ratio | 802.3 (FEC pre-/post-BER) | BERT and FEC counter at the receiver after the 480 km link |
| Laser safety class | ANSI Z136, IEC 60825 | Power at connector with ALS test; labelling and interlock verification |

The assignment rate is 25 Gb/s, whereas the Ethernet lane rate is 25.78125 GBd. Section 3.2 showed that the bandwidth requirement rises only to 18.05 GHz, still within 20 GHz. The IEEE PHYs are defined for fixed unamplified reaches and spacings, so they are used here as the source of the transmitter-parameter definitions (power, ER, SMSR, eye quality), while the grid and long-haul performance come from the ITU-T recommendations.

## 5.5 Integration approaches and system influence

Table 11: Discrete, monolithic InP and Si-photonics (hybrid laser) implementations

| Aspect | Discrete (TOSA/butterfly ×8) | Monolithic InP PIC | Si-photonics + hybrid laser |
| --- | --- | --- | --- |
| Footprint | Largest: 8 packages, 8 TECs | Small: 8 lasers + modulators + AWG on one chip | Smallest passive circuit; lasers bonded on |
| Power consumption | Highest: 8 TECs, 8 drivers | Low: one common TEC, shared package | Low: but heaters for wavelength control |
| Coupling loss | Several optical interfaces (lens, isolator) | Fewest: on-chip waveguides, one fibre coupling | Laser-to-Si and fibre-to-Si losses (several dB) |
| Output power / efficiency | Highest per laser (≈ 8.4 mW) | High (InP gain medium, SOA available) | Lower: loss in coupling and modulator |
| Wavelength accuracy | Each laser individually selected and tuned | Grating pitch tolerance: needs per-lane heater trim | Good passive control, laser still needs tuning |
| Reliability | Many joints, more failure points | Few joints, mature InP reliability | Bonding interface still maturing |
| Scalability | Poor: linear in cost and size | Good up to ≈ 8–16 channels | Excellent at high volume and with electronics (co-packaging) |

Integration affects system performance directly. Every on-chip connection removes a coupling interface (dB of loss and a reflection source), a shared TEC lowers power and heat, and a lithographic AWG fixes the channel spacing precisely. The cost is that, for a DFB array, all eight lasers share one chip temperature, so their grating-defined wavelengths must be matched on the grid to better than ±0.1 nm. This is why per-lane micro-heater trim and a wavelength locker are included in the design.

DESIGN DECISION 4 – CREATE THE INTEGRATED TRANSMITTER IMPLEMENTATION I recommend a monolithic InP photonic-integrated transmitter (8 × DFB + EAM with integrated AWG multiplexer on a common TEC, with per-lane heater trim and a wavelength locker), with dispersion-compensated, direct-detection nonlinear management and no full-link DBP because the InP platform natively provides the 1550 nm DFB (8.4 mW), the 20 GHz+ chirp-free EAM and the AWG on one chip, which removes the coupling loss that would consume the 0.23 dB power-budget margin and reduces eight TECs to one. Soliton transmission (N = 0.17, 68.4 mW needed) is infeasible, and DBP is unsuitable for an IM/DD, eight-channel link, so nonlinearity is controlled by launch-power management, SBS dithering and optical dispersion compensation. A Si-photonics hybrid design becomes preferable if lane counts or co-packaging with electronics grow beyond this design.

# STAGE 6 – VERIFY AND CRITICALLY EVALUATE THE COMPLETE TRANSMITTER SOLUTION

Table 12: Final verification of the transmitter solution against the original requirement

| Requirement | Specified / Target | Achieved | Satisfactory? |
| --- | --- | --- | --- |
| Optical source selection | 1550 nm, single-mode, SMSR ≥ 30 dB, narrow line | DFB: λ = 1.24/0.80 = 1550 nm, ηint = 0.80, SMSR 35–50 dB | Yes |
| Modulation bandwidth | B ≈ 0.7 × 25 = 17.5 GHz | 20 GHz; margin 2.5 GHz (14 %); 1.95 GHz at 25.78 GBd | Yes (thin margin) |
| Output power | ≈ 2 mW (+3 dBm) per channel at launch | 8.4 mW (+9.24 dBm) CW; 2.1 mW (+3.24 dBm) after 6 dB losses | Yes (0.23 dB margin) |
| Spectral / chirp stability | Low chirp; fits 0.8 nm pitch | External modulation: α ≈ 0–1; NRZ lobe ≈ 0.4 nm | Yes |
| Wavelength stability | ±0.1 nm over 40 °C | Uncontrolled 4.0 nm; TEC ±0.1 °C gives ±0.01 nm (limit ±1 °C) | Yes (10× margin) |
| SPM / XPM / FWM | φNL small; crosstalk small | φNL = 0.055 rad/span, 0.330 rad (6 spans); XPM walk-off 1088 ps/span; FWM ≈ –64.5 dB per product | Yes |
| SBS | Pch < Pth | Raw: 1.59 mW < 2 mW (violated by 1.0 dB); with 200 MHz dither, 17.5 mW | Yes, with dithering |
| SRS | Ptot < Pth | 16 mW vs 1.008 W (1.6 %) | Yes |
| Eye safety | Below limit or controlled | +12.04 dBm total vs assumed +10 dBm Class 1; ALS + controlled access | Yes, as controlled-access |
| Soliton / dispersion condition | Decide feasibility | LD = 11.25 km, P0 = 68.4 mW, N = 0.171: infeasible; dispersion-managed NRZ adopted | Yes (soliton correctly rejected) |
| Integration / DSP / standards | Compact, scalable, G.957 / 802.3 / Z136 verifiable | Monolithic InP PIC; DBP rejected; compliance tests tabulated (Table 10) | Yes |

Consistency with the requirement. The verification shows that the calculated results are consistent with the original specification at every stage: the 1550 nm source supplies sufficient power with a single longitudinal mode, the modulation bandwidth exceeds the NRZ requirement, the wavelength drift is reduced to a tenth of the allowed tolerance, and the accumulated SPM, the SRS, and the XPM/FWM penalties are small. The two items that required action are SBS (violated by 1.0 dB without dithering) and total power versus the assumed eye-safety limit. The weaknesses are equally visible: the bandwidth margin (2.5 GHz) and the power margin (0.23 dB) are small, and both depend on assumed values (6 dB of losses, modulator bandwidth, SBS bandwidth). These are the parameters to be measured first on prototype hardware.

# FINAL TRANSMITTER DESIGN – INTEGRATED ENGINEERING SOLUTION

## F.1 Final parameter and design table

Table 13: Final design parameters of the 8-channel DWDM transmitter

| Parameter | Adopted value |
| --- | --- |
| Optical source | 1550 nm InGaAsP/InP DFB laser per lane, 8 lanes (ηint = 0.80, Eg = 0.80 eV) |
| Channel grid | 193.1 – 193.8 THz at 100 GHz: 1552.52, 1551.72, 1550.92, 1550.12, 1549.32, 1548.51, 1547.72, 1546.92 nm |
| Bias / threshold | I = 40 mA, Ith = 12 mA (excess 28 mA) |
| CW output power per laser | 8.4 mW = +9.24 dBm (ηs = 0.3 mW/mA) |
| Power per channel at launch | ≈ 2.1 mW (+3.2 dBm) after ≈ 6 dB losses; 16 mW (+12.0 dBm) total for 2 mW/channel |
| Modulation | External, electro-absorption modulator integrated with DFB; 25 Gb/s NRZ |
| Bandwidth | 20 GHz available vs 17.5 GHz required (2.5 GHz margin) |
| Wavelength control | TEC set-point 25.0 °C with ±0.1 °C stability (±0.01 nm) + etalon locker + micro-heater trim |
| Drift protection | Uncontrolled 4.0 nm (5 channels) over 40 °C reduced to ±0.01 nm; limit ±1 °C |
| Nonlinear control | SBS dither ≥ 100 MHz (≈ 200 MHz linewidth → threshold ≈ 17.5 mW); per-span DCF (≈ –1360 ps/nm); launch ≈ 2 mW/channel |
| Nonlinear results | Leff = 21.17 km; φNL = 0.055 rad/span, 0.330 rad (6 spans); SRS threshold 1.008 W |
| Eye safety | Total +12.04 dBm exceeds assumed +10 dBm Class 1 limit: ALS (G.664), access control and labelling |
| Soliton / DSP | LD = 11.25 km, N = 0.171, soliton infeasible; DBP not used; receiver FFE/MLSE + FEC |
| Integration | Monolithic InP PIC with 8 DFB-EAM, AWG mux, optional SOA, common TEC |
| Standards | ITU-T G.957, G.694.1; IEEE 802.3bs/bm; ANSI Z136 (Z136.2) with ITU-T G.664 |

## F.2 Transmitter block diagram

Figure 4: Block Diagram of the Proposed 8-Channel DWDM Transmitter

Each of the eight lanes is a DFB laser biased at 40 mA CW (8.4 mW), followed by an electro-absorption modulator driven by a 25 Gb/s NRZ stream. The eight modulated carriers are combined by an on-chip 100 GHz AWG multiplexer, optionally boosted by an SOA and an EDFA, and sent into six 80 km spans, each followed by an EDFA and per-span DCF. A bias controller applies the 100 MHz SBS dither, a monitor photodiode and APC loop hold output power and extinction ratio, and a thermistor, etalon locker and TEC driver hold the chip at 25.0 ± 0.1 °C with micro-heater trim for each lane. An ALS circuit shuts the transmitter down on a fibre break.

## F.3 How the design choices work together

- Source → modulation. The DFB laser provides the single mode and 8.4 mW, and running it CW into an EAM keeps its chirp, bandwidth and wavelength free of the data pattern.

- Modulation → dispersion. The nearly transform-limited 0.2 nm spectrum makes the 1360 ps/nm per span a compensable, deterministic quantity, which per-span DCF removes. This also keeps XPM and FWM phase-mismatched.

- Wavelength control → crosstalk. ±0.01 nm stability keeps the 0.4 nm NRZ lobes centred in their 0.8 nm slots, protecting both the adjacent-channel crosstalk budget and the ±0.1 nm limit.

- Power → nonlinearity and safety. 2 mW per channel gives OSNR ≈ 32.2 dB and φNL = 0.330 rad, with SBS dithering handling the one violated threshold and ALS handling the eye-safety margin.

- Integration → margins. Monolithic InP removes coupling interfaces that would eat into the thin 0.23 dB power margin, and the common TEC cuts power and complexity across eight lanes.

- Soliton/DSP rejected. N = 0.171 and the lack of coherent detection show that neither is a valid route, and the design is correctly quasi-linear and dispersion-managed.

# FINAL ENGINEERING DECISION

WOULD YOU DEPLOY THE TRANSMITTER SOLUTION YOU CREATED? YES – with the conditions below. Technical justification. Source: the 1550 nm DFB (ηint = 0.80, 8.4 mW, SMSR ≥ 30 dB) matches the link window and is the only candidate that satisfies power, spectral purity and speed at once. Modulation: the 17.5 GHz requirement is met with 2.5 GHz margin, and external modulation removes the αH ≈ 4 chirp that would be damaging over 80 km spans. Stability: the 4.0 nm uncontrolled drift is reduced to ±0.01 nm, ten times inside the ±0.1 nm tolerance. Nonlinearity: φNL = 0.330 rad over six spans, SRS at 1.6 % of threshold, and strongly suppressed XPM/FWM mean that only SBS (1.59 mW against 2 mW) needs active control, which ≥ 100 MHz dithering provides with a 9.4 dB margin. Soliton/compensation: N = 0.171 and a needed 68.4 mW make solitons unsuitable, and DBP does not apply to an IM/DD eight-channel link, so dispersion-managed NRZ is the right architecture. Standards/integration: parameters from G.957 and IEEE 802.3bs/bm are verifiable on the proposed test set, and the monolithic InP implementation gives the smallest coupling loss and power. Conditions for deployment. (1) The +12 dBm aggregate must be operated as a controlled-access, ALS-protected installation (or reduced to 1.25 mW per channel if Class 1 is mandatory). (2) The assumed 6 dB modulator, coupling and AWG loss, the ≥ 20 GHz EAM bandwidth and the Brillouin bandwidth must be confirmed by measurement before volume deployment, since the power (0.23 dB) and bandwidth (2.5 GHz) margins are thin. (3) Per-lane wavelength accuracy of the InP array on the common TEC must be verified to ±0.1 nm. If any of these tests fail, the fall-backs are a higher bias (≈ 45 mA gives 9.9 mW) and the discrete TOSA implementation.